Colorimeter

Software Operation Manual V1.32

Please read this manual carefully before use.

- Colorimeter Software Introduction

ColoriMeter is the support software for Colorimeter. This software can realize color data reading and writing, historical data reading, online color difference measurement, exporting data in EXCEL format, and report generation, etc.

The device is equipped with a USB Type C communication interface. Connect the device with the computer by a USB cable, and start ColoriMeter, you can read the data in the device.

Particularly, the software has functions of report generation and printing. The software can automatically generate and print reports of measurement data. If a PDF printer is installed on your computer, you can print an electronic version of the report in PDF format. It is convenient to record and store measurement data.

You don't have to manually install the USB driver. Just connect the device with the computer, and you can realize online operation (when you connect the device with the computer for the first time, an alert will ask you to restart the computer to automatically load the driver). Currently the software supports Windows. Computer hardware configuration recommended 6th Generation Intel® Core™ i3 Processors, RAM greater than 4G.

- Software Installation

This software does not require installation. You can start the software by clicking the exe file. You should run the program as an administrator before your first use of the software.

Setup method: Move the mouse to "Colorimeter.exe", click the right mouse button, click "Properties", then select "Compatibility", check "Run as administrator", and click "Confirm".

If you need to copy the software, you should copy the entire folder.

- Hardware Connection

Connect the device and computer by plugging the cable into the USB ports thereof respectively. After connection, “USB Connected” appears at the upper right corner on the device display.

- Software Interface

- USB Connection Status Indicator
Normal device connection: "Green".
Abnormal or no device connection: "Gray".

- Device Information
It includes the SN and Firmware Version.

- Language Selection
"中文": click the button to switch the interface to Chinese.
"English": click the button to switch the interface to English.

- ΔE parameter
It contains the ΔE formula and the reading and setting of the ΔE threshold.

- Count
It includes the number of PASS, NG, and Total.

- Function Buttons
”Clear”: clear all data in the data list. 
”Export Excel”: Export the data in the list in Excel format. 
”Import Data”: Re-import the Excel data exported by the software into the list.

- Data Display Area
It contains the Test Name, Time, Standard color measurement, Sample color measurement, ΔE value, and PASS/NG in the color difference comparison records.

- Device Data Transmission
“Read mycolor”: Export My Color in the device in Excel format.
“Write myColor”: write My Color exported by the software into the device, so that multiple devices of the same model can share the color. 
“Read color difference records”: import the historical records of the color difference to the list.  
“Write stdcolor”: update the color library in the device.

- Color Difference Measurement
“Scan Standard”: connect with the computer to measure the standard color. 
“Scan Sample”: connect with the computer to measure the sample color. 
Display the color deviation indicator and QC qualification parameters.

- Software Operation

3.1 Color difference measurement

After the device connects with the computer, start the software to carry out online color difference comparison. Click "Scan Standard" or "Scan Sample" to obtain the corresponding color data. When there are data for the standard and sample colors, the color deviation indicator and QC qualifications will display, and a record will be generated in the list.

Shortly press the button on the device, or press "Enter" on the keyboard may also trigger measurement.

3.2 Parameter setting

Click the "Parameter Setting" button to pop up the parameter setting interface, select the ΔE formula and input the threshold, and then click "OK" to download the parameter settings into the device.

If the parameter setting is successful, the following two items will be recalculated and re-qualified according to the set ΔE formula and threshold.

- ΔE value and qualification in color difference measurement.

- ΔE value and qualification in the list, and counting of PASS.

3.3 Device data transmission

- Read My Color to the computer.

Click "Read My Color" to pop up the storage path selection box. After setting, click "Save", the software will export My Color data from the device. After completion, the software will prompt: "Read My Color Success!"

- Write My Color to the device

Click "Write MyColor" to pop up the file path selection box. After selecting the file to be written into the device, click "Open", the software will write the color data of the Excel file into the device. After completion, the software will prompt: "Write My Color Success!"

Note: The written content must be the color data exported through the software. Colors stored in the device will be replaced after new colors are written.

- Read color difference record to the list

Click "Read Color Difference Record" to the list. If there are no data in the list, the device data will be imported to the list. If there are measurement data in the list, the software will prompt: "Import will clear the list, store data first?" Select "YES" to pop up the storage path selection box, and export the data in the list. Select "NO", clear the list first and then import the device data.

- Write standard color library to the device

Click "Write Stdcolor" to load the color library to the device, the software pops up the file path selection box. Select the file to be written into the device, and click "Open". The software will write the standard color library of the Excel file into the device. After completion, the software will prompt: "Write stdcolor library success!"

Note: The standard color library is provided by the supplier.

3.4 Operations in the list

- Clear Data

Click "Clear" to clear the data in the list, qualification counts and the data in the color difference measurement interface.

- Export to Excel

Click "Export Excel" to pop up the storage path selection box. After setting, click "Save" to export the data from the list in the Excel format. After completion, the data in the list will be cleared.

- Import Data

Click "Import Data". If there are no data in the list, the device data will be imported to the list. If there are measurement data in the list, the software will prompt: "Import will clear the list, store data first?" Select "YES" to pop up the storage path selection box, and export the data in the list before data import. Select "NO", clear the list first and then import the device data.

Note: The imported content must be the color difference data exported by the software; otherwise the software will make mistakes.

- Edit Name

The record generated by color difference measurement is named by default. If you need to change the name, you can double-click the name of the record for modification.

3.5 Report generation

Click "Print" of a single record in the list to generate the report as follows.

For header information input, click "Information" to enter the head information of the Company, Tester, and Auditor.

Company: Shenzhen Linshang Technology Co., Ltd.

Website: www.linshangtech.com

Service Hotline: +86 0755 86263411

Email: sales21@linshangtech.com